FPGA-Based Acoustic Drone Detection

Using a Rate-Coded, Spike-Driven Neuromorphic Accelerator

# 1. Objective

The goal of this project is to design, implement, and verify a hardware accelerator capable of classifying short audio clips as either “drone” or “ambient” (background noise), running entirely on a low-cost FPGA (Terasic DE2-115, Cyclone IV) rather than a CPU or GPU.

The project takes a neural network already trained in Python and converts it, module by module, into synthesizable Verilog RTL — with every module independently verified against the original software implementation before being trusted.

# 2. Background and Motivation

- Unauthorized drone activity is a growing security and privacy concern in restricted airspace.

- Acoustic signatures offer a low-cost, passive detection method that does not require radar or visual line-of-sight.

- FPGA implementation allows low-power, real-time, always-on operation without a general-purpose processor.

# 3. System Overview

The system is organized into two stages: an offline software stage (feature extraction and model training, done once in Python) and an always-on hardware stage (audio-derived data classified in real time on the FPGA).

| Stage | Where it runs | Purpose |
| --- | --- | --- |
| Feature extraction (STFT) | Software (Python) | Converts a raw waveform into a 32×20 time-frequency feature map |
| Rate encoding | Software (Python), matched exactly in hardware | Converts the feature map into a 50-timestep spike train |
| Model training | Software (Python) | Learns a 640→128→2 classifier (sigmoid hidden layer, softmax output) |
| Spike-gated inference | Hardware (FPGA, Verilog RTL) | Reproduces the trained model's decision using event-driven, fixed-point hardware |

# 4. Why “SNN” but No LIF Neuron

The project is described as a Spiking Neural Network, but it is important to state precisely what is and is not spiking, to avoid overclaiming.

- The trained classifier itself is a standard sigmoid/softmax network — it has no membrane potential, firing threshold, or leak term.

- This follows a published technique (Diehl et al., 2015) of training an ordinary neural network on rate-equivalent data, then deploying the trained weights on spike-driven hardware.

- What is genuinely spiking: the input encoding (a real, deterministic hardware random-number generator drives 50 timesteps of stochastic spikes) and the compute style (neurons that do not fire consume zero computation cycles — a real hardware efficiency, not just terminology).

Accurate description: a rate-coded, spike-driven neuromorphic accelerator.

# 5. Hardware Architecture

The datapath is organized as a sequence of independently verified modules:

| Module | Function |
| --- | --- |
| LFSR (pseudo-random source) | Deterministic spike generator, bit-matched to the Python reference |
| Rate Encoder | Converts stored thresholds into a 50-timestep spike stream |
| Spike-Gated Accumulator | Sums trained weights into the hidden layer, only for neurons that fire |
| Fixed-Point Rescale + Sigmoid LUT | Reproduces the trained activation function in fixed-point hardware |
| Output Layer | Dense weighted sum and final drone/ambient decision |
| Top-Level Controller | Sequences the full classification behind a single start/done interface |

# 6. Verification Methodology

Every hardware module was checked bit-for-bit against an independently written software reference, using real recorded audio samples at every stage — not artificial test data. This uncovered and allowed correction of several real design errors before they could affect the final result, including a timing race condition, an accumulator logic error, and — most significantly — a missing data-normalization step that was initially overlooked.

This last issue is worth highlighting as an example of the verification process working as intended: when tested on a batch of real samples, accuracy was found to be only 60.4%, far below the expected value. Systematic investigation traced this to a data-normalization step present in the training pipeline but missing from the hardware's mathematical model. Correcting it — without changing any hardware architecture — restored accuracy to 97.9%, confirmed by re-running the corrected design in simulation.

# 7. Results

| Metric | Result |
| --- | --- |
| Hardware vs. software bit-exact match | 48 / 48 real audio samples |
| Classification accuracy vs. ground truth | 47 / 48 (97.9%) |
| FPGA synthesis | 0 errors (Quartus II, Cyclone IV EP4CE115) |
| Timing closure | Passes 50 MHz with positive margin in every analyzed condition |
| Resource utilization | ≈ 85% of available logic cells |

# 8. Demonstration Interface

For demonstration purposes, since a live real-time audio front end was outside the project's scope, four pre-recorded and pre-verified audio samples are loaded into the FPGA at compile time. Two physical push-buttons select which of the four samples is classified; two LEDs indicate the result (drone or ambient).

# 9. Conclusion and Learning Outcomes

- Demonstrated a complete, working path from a trained software model to verified, synthesizable hardware.

- Applied rigorous, evidence-based verification — every design claim is backed by a reproducible test against real data, not assumption.

- Encountered and correctly diagnosed real hardware/software integration issues, including a significant accuracy-impacting bug, through systematic root-cause analysis rather than trial and error.

- Achieved working timing closure on real target hardware at a genuine operating frequency.

# Appendix: Project Timeline Summary

The project proceeded through the following phases: (1) analysis of the existing software signal-processing and training pipeline; (2) hardware architecture design; (3) module-by-module RTL implementation, each verified independently against real data; (4) discovery and correction of a significant accuracy-impacting integration bug; (5) full-batch verification at the RTL simulation level; (6) FPGA synthesis and timing closure; (7) demonstration interface design and hardware bring-up.